| # Canadian Natural Resources Limited # GENERAL PRESSURE VESSEL INFORMATION |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| District: Ft St John B.C. |  |  |  |  | Skid No. |  |  |  |  |  |
| ### Facility: Fox Compressor Station |  |  |  |  | Location (LSD): d – 47 – D / 94 – A – 15 |  |  |  |  |  |
| Vessel Name & Equipment Number: Glycol Contactor |  |  |  |  |  |  |  |  |  |  |
| ### Orientation: Vertical |  |  |  |  |  |  |  |  |  |  |
| ### Status: Operating |  |  |  |  | Regulatory Inspection |  |  |  |  |  |
| PRESSURE VESSEL NAMEPLATE DATA |  |  |  |  |  |  |  |  |  |  |
| Registration Number A152541 |  |  |  |  |  | CRN Number F 1504.1 |  |  |  |  |
| Vessel serial number: F2407 – 1 |  |  |  |  |  | Size: 24 in. OD x 282 in. S/S |  |  |  |  |
| Shell thickness: 38.9 mm |  |  |  |  |  | Shell material: SA 106 B |  |  |  |  |
| Head thickness: 27.0 mm |  |  |  |  |  | Head material: SA 516 70 |  |  |  |  |
| Tube wall thickness: |  |  |  |  |  | Tube material: |  |  |  |  |
| Tube diameter: |  |  |  |  |  | Tube length: |  |  |  |  |
| Channel thickness: |  |  |  |  |  | Channel material: |  |  |  |  |
| Design pressure | Shell: 9929 Kpa |  |  |  |  | Operating pressure |  | Shell: |  |  |
|  | Tubes: |  |  |  |  |  |  | Tubes: |  |  |
| Design Temp. | Shell: 60 deg C |  |  |  |  | Operating temperature |  | Shell: |  |  |
|  | Tubes: |  |  |  |  |  |  | Tubes: |  |  |
| X-ray: RT 1 |  |  |  |  |  | Heat treatment: HT |  |  |  |  |
| Code parameters: ASME VIII / Div 1 |  |  |  |  |  | Coated: No |  |  |  |  |
| Manufacturer: CESSCO |  |  |  |  |  | Year built: 1981 |  |  |  |  |
| Corrosion allowance: 3.2 mm |  |  |  |  |  | Manway: No |  |  |  |  |
| # PRESSURE SAFETY VALVE NAMEPLATE DATA |  |  |  |  |  |  |  |  |  |  |
| PSV Tag no. | Manufacture | Model | Serial number |  |  |  | Set Pressure |  | Capacity | Size |
| Serv By | Date | Block Valve | CRN number |  |  |  | Code Stamp |  | Location |  |
| Unified | Dec 03 |  |  |  |  |  |  |  |  |  |
| # SERVICE CONDTIONS-INDICATE ALL THAT APPLY |  |  |  |  |  |  |  |  |  |  |
| Sweet X | Sour |  |  | Oil |  |  |  | Gas X |  | Water X |
| Amine | LPG |  |  | Condensate |  |  |  | Air |  | Glycol X |
| Other (Describe): |  |  |  |  |  |  |  |  |  |  |

- Inspection Interval ________________________________________PSV Service Interval_________________________________

- (Determined by MIC in conjunction with Chief Inspector following guidelines of  CNRL’s Owner-User Inspection Program)

Reports reviewed and accepted by:

- Mechanical Integrity Coordinator________________________________________________Date__________________________

- Fill out all forms as completely as possible. All information is important!  Use back of sheets to record additional information or sketch if required.

- Copy of report to be filed by MIC at site,  and copy sent to Chief Inspector

### Glycol Contactor

| External Inspection Items | G | F | P | N/A | Comments |
| --- | --- | --- | --- | --- | --- |
| Insulation: Verify sealed around manways, nozzles, no damage present, and there is no egress of moisture. |  |  |  | X | Vessel is not insulated. |
| External Condition: Assess paint condition, areas peeling, record any corrosion, damage, etc (record location, size and depth of corrosion or damage) | X |  |  |  | No loose or flaking paint. |
| Leakage: Record any leakage at flanges, threaded joints, weep holes on repads, etc. | X |  |  |  | No leaks detected. |
| Skirt: Assess condition of paint, fire protection, concrete. Look for corrosion, buckling, dents, etc. Look at vessel surface area near supports. Verify no signs of leakage at attachment to vessel and attachment welds are acceptable. Ground wire attached? | X |  |  |  | Contactor is mounted firmly to skid deck. Skid Package is grounded. |
| Anchor Bolts: Hammer tap to ensure secure. Look for cracking in treads or signs of deformation. | X |  |  |  | Firmly bolted. |
| Concrete foundation: Check for cracks, spalling, etc. |  |  |  | X |  |
| Ladder / Platform: Describe general condition, ensure support is secure to vessel, describe any hazards. |  |  |  | X |  |
| Nozzle: Assess paint, look for leakage, and ensure stud threads are fully engaged. Record any damage, deflection, etc. Are nozzles gusseted? | X |  |  |  | Good condition, no leaking detected. |
| Gauges: Ensure gauges are visible, working, no leakage, and suitable for range of MAWP/ Temp. | X |  |  |  | Firmly attached and within operational parameters for service. |
| External Piping: Ensure pipe is well supported. All clamps, supports, shoes, etc. in place. Look for evidence of structural overload, deflection, etc. Paint condition, external corrosion? | X |  |  |  | Firmly supported, no distortion or deflection. |
| Valving: Ensure no leaks are visible. Valves are properly supported and chained if necessary. | X |  |  |  | Firmly attached, no leaks. |
| PSV: Ensure PSV is set at pressure at or below that of vessel. | X |  |  |  | Located on Top head, set at MAWP of vessel. |
| ### NDE methods: Was UT/ MPI done on vessel (MI coordinator to review results) | X |  |  |  | Ultrasonic thickness survey carried out – No items of concern. |
| Recommendations or corrective actions : Vessel is Fit for Service or describe corrective actions required) (MIC to review corrective actions with Operations, discuss with Chief Inspector where necessary, and get remedial action implemented) Recommendations: - No recommendations at this time – return vessel to service. Summary: This vessel is in good condition, ultrasonic corrosion survey carried out – no metal thickness detected below nominal minus corrosion allowance. Vessel is fit for service. |  |  |  |  |  |

Inspected By:  D. Wiedman                                                                                                                                          Date: Feb 2004

Glycol Contactor